# Dover Gardens project brief and specification.

## LNB decided early-on in its development that all its properties and sites would be designed to be sustainable and with good accessibility so that families with young children through to people with mobility aides can move with ease in their own homes and through their surroundings.  In LNB developments:

## We aim to achieve a minimum 7 star energy efficiency rating by applying passive solar design principles and incorporating energy saving features and technology.

## When designing, we refer to the Liveable Homes standards to ensure sufficient circulation space, trip hazard reduction and long term sustainability.

## We consider sustainability of materials with regard to toxicity, durability, embodied energy and recyclability when selecting building products and appliances.

By putting future residents and community front and centre in the development process, our projects are shaped by their diverse interests. We involve people in the final designs and selections and discuss opportunities for shared resources and community gardens.  Our landscapes are based on permaculture principles.

## Walls (external Walls: 190mm)

External: 75mm Hebel Veneer

Wall Frames: Timber  (90mm wall frames)

Internal linings: 10mm Plasterboard

Floor & Roof

Roof Material: Colorbond custom orb (light colour: Dune, surfmist, shale grey or paperbark  Reflective Foil to Roof:

Insulation

Internal Walls: R2.0

External Walls: R2.5

Ceiling: Up to R6.0

Garage Ceiling: R5.0

Reflective Foil External wall wrap

Glazing

General Windows:  Aluminium Residential frames with 4mm Energy tech Low E Glass generally.

Front Windows: Double glazed Low E Glass.

Floor Coverings:

Carpet: TBC

Floating Timber Flooring

Tiles To All Wet Areas (walls and floors)

Ventilation

Ceiling fans throughout

Services

Airconditioning: – Options TBC

Artificial Lighting: 14W LED Downlights throughout

Solar Electricity:  2kw Photovoltaic system

Heated Water Service: Electric Boosted Solar HWS or Heat Pump HWS

WATER SAVING

In-ground Rainwater tanks (per house) 1 x 22,000 ltr and plumbed into dwelling. Auto mains feed into tank as required.

PASSIVE and ‘liveable’ DESIGN

Windows to the North and West to have up to 900mm wide extended eaves overhang

Ceiling height is 2700mm

A mix of high level windows and high level celestory windows and aligned to channel prevailing summer breezes through living spaces.

Promotion of bright living areas, circulation to space compartmentalized to save energy leakage (ie; door locations to hallways, large windows to living areas and all spaces designed for good circulation)

Encourages the use of inside/outside spaces for entertaining as well as circulation to garden areas to south western boundary which maximizes the daytime sun movement and promotes a self sufficient  garden area.

Draught seals to all doors.  Doorways are wide to accommodate accessibility standards.

Disability access – Ply linings installed to wet areas and power points + light switches set at industry standard heights.

Openable high level windows to front entry area (promoting security and picking up breezes for excellent cross ventilation all day round in summer).

Kitchen, main bathroom, toilet, bedroom and living rooms are designed with reference to circulation spaces.  Front entry is designed to provide easy access for people with mobility issues.

OTHER  FEATURES

All supplied appliances highly energy efficient – TBC.

Excavated topsoil from slab prep to be stockpiled on site rather than dumped off site – used to build up garden areas and forming garden beds for self sufficient garden areas.